Sediment Approach

- Toxics have varying solubility in water which affects their movement.

- Water soluble toxics (such as, diazinon, atrazine, & some heavy metals) can be transported in surface water or ground water

- Less soluble toxics can bind to soil particles (such as, Pyrethroids, DDT & PCBs )

- Erosion and transport of soil to waterbodies can carry soil-bound toxic chemicals

- But, certain types and delivery of sediment is important for proper stream function

Episodic Sediment Delivery: Benefits

- Sediment delivery to bodies of water is a key part of natural ecosystems

- Slides on forested lands deliver key components to aquatic ecosystems

  - Large woody debris (LWD)

  - Gravel and cobbles

- Slides from mature or burned forests are rare occurrences

  - Once every ~300 years in larger streams, less often in smaller streams

- Short-term negative impacts with long-term gains in ecosystem productivity

Chronic Sources of Fine Sediment: Potentially Harmful

- Chronic sources like road runoff create constant stress on aquatic ecosystems

- These sources lack the legacy that creates long-term benefits  (no LWD or gravel)

Sediment Delivery and Toxics

- Nonpoint Sources of fine sediment and toxics:

  - Roads (Heavy metals, PAHs, other petroleum products)

  - Urban Stormwater

  - Agricultural lands (Pesticides, petroleum products)

  - Private forest operations (Pesticides, road runoff)