TABLE 1 OAR 340-200-0020 SIGNIFICANT AIR QUALITY IMPACT

Pollutant; Averaging Time; Air Quality Area Designation. Class I; Class II; Class III. SO2 (µg/m3)*; Annual; 0.10; 1.0; 1.0. 24-hour; 0.20; 5.0; 5.0. 3-hour; 1.0; 25.0; 25.0. PM10 (µg/m3); Annual; 0.20; 0.2; 0.2. 24-hour; 0.30; 1.0; 1.0. PM2.5 (µg/m3); Annual; 0.06; 0.3; 0.3. 24-hour; 0.07; 1.2; 1.2. NO2 (µg/m3); Annual; 0.10; 1.0; 1.0. CO (mg/m3)**; 8 hour; ---; 0.5; 0.5. 1-hour; ---; 2.0; 2.0. * micrograms/cubic meter **milligrams/cubic meter.

| Table 2 OAR 340-200-0020 SIGNIFICANT EMISSION RATES |  |
| --- | --- |
| Pollutant | Emission Rate |
| Greenhouse Gases (CO2e) | 75,000 tons/year |
| Carbon Monoxide | 100 tons/year |
| Nitrogen Oxides (NOX) | 40 tons/year |
| Particulate Matter | 25 tons/year |
| PM10 | 15 tons/year |
| Direct PM2.5 | 10 tons/year |
| PM2.5 precursors (SO2 or NOx) | 40 tons/year |
| Sulfur Dioxide (SO2) | 40 tons/year |
| Volatile Organic Compounds (VOC) | 40 tons/year |
| Ozone precursors (VOC or NOx) | 40 tons/year |
| Lead | 0.6 ton/year |
| Fluorides | 3 tons/year |
| Sulfuric Acid Mist | 7 tons/year |
| Hydrogen Sulfide | 10 tons/year |
| Total Reduced Sulfur (including hydrogen sulfide) | 10 tons/year |
| Reduced sulfur compounds (including hydrogen sulfide) | 10 tons/year |
| Municipal waste combustor organics (measured as total tetra- through octa- chlorinated dibenzo-p-dioxins and dibenzofurans) | 0.0000035 ton/year |
| Municipal waste combustor metals (measured as particulate matter) | 15 tons/year |
| Municipal waste combustor acid gases (measured as sulfur dioxide and hydrogen chloride) | 40 tons/year |
| Municipal solid waste landfill emissions (measured as nonmethane organic compounds) | 50 tons/year |

| Table 3 OAR 340-200-0020 SIGNIFICANT EMISSION RATES FOR THE MEDFORD-ASHLAND AIR QUALITY MAINTENANCE AREA |  |  |
| --- | --- | --- |
| Air Contaminant | Emission Rate |  |
|  | Annual | Day |
| PM10 | (5.0 tons) | (50.0 lbs.) |

| TABLE 4 OAR 340-200-0020(31) De Minimis Emission Levels |  |
| --- | --- |
| Pollutant | De minimis (tons/year, except as noted) |
| Greenhouse Gases (CO2e) | 2,756 |
| CO | 1 |
| NOx | 1 |
| SO2 | 1 |
| VOC | 1 |
| PM | 1 |
| PM10 (except Medford AQMA) | 1 |
| PM10/PM2.5 (Medford AQMA) | 0.5 [5.0 lbs/day] |
| Direct PM2.5 | 1 |
| Lead | 0.1 |
| Fluorides | 0.3 |
| Sulfuric Acid Mist | 0.7 |
| Hydrogen Sulfide | 1 |
| Total Reduced Sulfur (including hydrogen sulfide) | 1 |
| Reduced Sulfur | 1 |
| Municipal waste combustor organics (Dioxin and furans) | 0.0000005 |
| Municipal waste combustor metals | 1 |
| Municipal waste combustor acid gases | 1 |
| Municipal solid waste landfill gases | 1 |
| Single HAP | 1 |
| Combined HAP (aggregate) | 1 |

| TABLE 5 OAR 340-200-0020(56) Generic PSELs |  |
| --- | --- |
| Pollutant | Generic PSEL (tons/year, except as noted) |
| GreenhouseGases (CO2e) | 74,000 |
| CO | 99 |
| NOx | 39 |
| SO2 | 39 |
| VOC | 39 |
| PM | 24 |
| PM10 (except Medford AQMA) | 14 |
| PM10/PM2.5 (Medford AQMA) | 4.5 [49 lbs/day] |
| Direct PM2.5 | 9 |
| Lead | 0.5 |
| Fluorides | 2 |
| Sulfuric Acid Mist | 6 |
| Hydrogen Sulfide | 9 |
| Total Reduced Sulfur (including hydrogen sulfide) | 9 |
| Reduced Sulfur | 9 |
| Municipal waste combustor organics (Dioxin and furans) | 0.0000030 |
| Municipal waste combustor metals | 14 |
| Municipal waste combustor acid gases | 39 |
| Municipal solid waste landfill gases | 49 |
| Single HAP | 9 |
| Combined HAPs (aggregate) | 24 |