| # State of Oregon Department of Environmental Quality # Significant Air Quality Impact # Table 1 - OAR 340-200-0020 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| # Pollutant | # Averaging Time | # Air Quality Area Designation |  |  |  |
|  |  | # Class I | # Class II | # Class III |  |
| # SO2 (µg/m3)* | # Annual | # 0.10 | # 1.0 | # 1.0 |  |
|  | # 24-hour | # 0.20 | # 5.9 | # 5.0 |  |
|  | # 3-hour | # 1.0 | # 25.0 | # 25.0 |  |
| # PM10 (µg/m3) | # Annual | # 0.20 | # 0.20 | # 0.20 |  |
|  | # 24-hour | # 0.30 | # 1.0 | # 1.0 |  |
| # PM2.5 (µg/m3) | # Annual | # 0.06 | # 0.3 | # 0.3 |  |
|  | # 24-hour | # 0.07 | # 1.2 | # 1.2 |  |
| # NO2 (µg/m3) | # Annual | # 0.10 | # 1.0 | # 1.0 |  |
| # CO (mg/m3)** | # 8-hour | # --- | # 0.5 | # 0.5 |  |
|  | # 1-hour | # --- | # 2.0 | # 2.0 |  |
| # *micrograms/cubic meter # **milligrams/cubic meter |  |  |  |  |  |

| State of Oregon Department of Environmental Quality # Significant Emission Rates # Table 2 - OAR 340-200-0020 # -20-8010) |  |  |
| --- | --- | --- |
| # Pollutant | # Emission Rate |  |
| # Greenhouse Gases (CO2e) | 75,000 tons/year |  |
| # Carbon Monoxide | 100 tons/year |  |
| # Nitrogen Oxides (NOX) | 40 tons/year |  |
| # Particulate Matter | 25 tons/year |  |
| # PM10 | 15 tons/year |  |
| # Direct PM2.5 | 10 tons/year |  |
| # PM2.5 precursors (SO2 or NOx) | 40 tons/year |  |
| # Sulfur Dioxide (SO2) | 40 tons/year |  |
| # Volatile Organic Compounds (VOC) | 40 tons/year |  |
| # Ozone precursors (VOC or NOx) | 40 tons/year |  |
| # Lead | 0.6 ton/year |  |
| # Fluorides | # 3 tons/year |  |
| # Sulfuric Acid Mist | # 7 tons/year |  |
| # Hydrogen Sulfide | # 10 tons/year |  |
| # Total Reduced Sulfur (including hydrogen sulfide) | # 10 tons/year |  |
| # Reduced sulfur compounds (including hydrogen sulfide) | # 10 tons/year |  |
| # Municipal waste combustor organics (measured as total tetra- through octa- chlorinated dibenzo-p-dioxins and dibenzofurans) | # 0.0000035 ton/year |  |
| # Municipal waste combustor metals (measured as particulate matter) | # 15 tons/year |  |
| # Municipal waste combustor acid gases (measured as sulfur dioxide and hydrogen chloride) | # 40 tons/year |  |
| # Municipal solid waste landfill emissions (measured as nonmethane organic compounds) | # 50 tons/year |  |

| # State of Oregon Department of Environmental Quality # Significant Emission Rates for the Medford-Ashland Air # Quality Maintenance Area # Table 3 - OAR 340-200-0020 |  |  |  |
| --- | --- | --- | --- |
| # Air Contaminant | # Emission Rate |  |  |
|  | Annual | Day |  |
| # PM10 | (5.0 tons/year) | # (50.0 lbs.) |  |

| # State of Oregon Department of Environmental Quality # De Minimis Emission Levels # Table 4 - OAR 340-200-0020(33) |  |  |
| --- | --- | --- |
| # Pollutant | # De minimis (tons/year, except as noted) |  |
| # Greenhouse Gases (CO2e) | # 2,756 |  |
| # CO | # 1 |  |
| # NOx | # 1 |  |
| # SO2 | # 1 |  |
| # VOC | # 1 |  |
| # PM | # 1 |  |
| # PM10 (except Medford AQMA) | # 1 |  |
| # PM10/PM2.5 (Medford AQMA) | # 0.5 [5.0 lbs/day] |  |
| # Direct PM2.5 | # 1 |  |
| # Lead | # 0.1 |  |
| # Fluorides | # 0.3 |  |
| # Sulfuric Acid Mist | # 0.7 |  |
| # Hydrogen Sulfide | # 1 |  |
| # Total Reduced Sulfur (including hydrogen sulfide) | # 1 |  |
| # Reduced Sulfur | # 1 |  |
| # Municipal waste combustor organics (Dioxin and furans) | # 0.0000005 |  |
| # Municipal waste combustor metals | # 1 |  |
| # Municipal waste combustor acid gases | # 1 |  |
| # Municipal solid waste landfill gases | # 1 |  |
| # Single HAP | # 1 |  |
| # Combined HAP (aggregate) | # 1 |  |

| # State of Oregon Department of Environmental Quality # Generic PSELs # Table 5 - OAR 340-200-0020(60) |  |  |
| --- | --- | --- |
| # Pollutant | # Generic PSEL # (tons/year, except as noted) |  |
| # Greenhouse Gases (CO2e) | # 74,000 |  |
| # CO | # 99 |  |
| # NOx | # 39 |  |
| # SO2 | # 39 |  |
| # VOC | # 39 |  |
| # PM | # 24 |  |
| # PM10 (except Medford AQMA) | # 14 |  |
| # PM10/PM2.5 (Medford AQMA) | # 4.5 [49 lbs/day] |  |
| # PM2.5 | # 9 |  |
| # Lead | # 0.5 |  |
| # Fluorides | # 2 |  |
| # Sulfuric Acid Mist | # 6 |  |
| # Hydrogen Sulfide | # 9 |  |
| # Total Reduced Sulfur (including hydrogen sulfide) | # 9 |  |
| # Reduced Sulfur | # 9 |  |
| # Municipal waste combustor organics (Dioxin and furans) | # 0.0000030 |  |
| # Municipal waste combustor metals | # 14 |  |
| # Municipal waste combustor acid gases | # 39 |  |
| # Municipal solid waste landfill gases | # 49 |  |
| # Single HAP | # 9 |  |
| # Combined HAPs (aggregate) | # 24 |  |