DIVISION 228

REQUIREMENTS FOR FUEL BURNING EQUIPMENT AND FUEL SULFUR CONTENT

340-228-0020

Definitions

The definitions in OAR 340-200-0020, 340-204-0010 and this rule apply to this division. If the same term is defined in this rule and OAR 340-200-0020 or 340-204-010, the definition in this rule applies to this division.

(1) "Distillate Fuel Oil" means any oil meeting the specifications of ASTM Grade 1 or 2 fuel oils;

(2) "Residual Fuel Oil" means any oil meeting the specifications of ASTM Grade 4, 5, or 6 fuel oils.

NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the Environmental Quality Commission under OAR 340-200-0040.

[Publications: Publications referenced are available from the agency.]

Stat. Auth.: ORS 468 & 468A
Stats. Implemented: ORS 468.020 & 468A.025
Hist.: [DEQ 16, f. 6-12-70, ef. 7-11-70; DEQ 1-1984, f. & ef. 1-16-84; DEQ 4-1993, f. & cert. ef. 3-10-93; DEQ 3-1996, f. & cert. ef. 1-29-96]; [DEQ 37, f. 2-15-72, ef. 3-1-72; DEQ 4-1993, f. & cert. ef. 3-10-93]; [DEQ 37, f. 2-15-72, ef. 3-1-72; DEQ 4-1993, f. & cert. ef. 3-10-93]; DEQ 14-1999, f. & cert. ef. 10-14-99, Renumbered from 340-021-0005, 340-022-0005, 340-022-0050; DEQ 8-2007, f. & cert. ef. 11-8-07; DEQ 7-2011, f. & cert. ef. 6-24-11; Administrative correction, 2-6-12; DEQ 1-2012, f. & cert. ef. 5-17-12

Sulfur Content of Fuels

340-228-0120

Coal

(1) Except as provided in section (2), no person shall sell, distribute, use, or make available for use, any coal containing greater than 1.0 percent sulfur by weight.

(2)No person shall sell, distribute, use or make available for use any coal or coal containing fuel with greater than 0.3 percent sulfur and five percent volatile matter as defined in ASTM Method D3175 for direct space heating within the Portland, Salem, Eugene-Springfield, and Medford-Ashland Air Quality Maintenance Areas. For coals subjected to a devolatilization process, compliance with the sulfur limit may be demonstrated on the sulfur content of coal prior to the devolatilization process.

(3) Distributors of coal or coal containing fuel destined for direct residential space heating use shall keep records for a five year period which shall be available for DEQ inspection and which:

(a) Specify quantities of coal or coal containing fuels sold;

(b) Contain name and address of customers who are sold coal or coal containing fuels;

(c) Specify the sulfur and volatile content of coal or the coal containing fuel sold to residences in the Portland, Salem, Eugene-Springfield, and Medford-Ashland Air Quality Maintenance Areas.

[NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the Environmental Quality Commission under OAR 340-200-0040.]

[Publications: The publication(s) referred to or incorporated by reference in this rule are available from the agency.]

Stat. Auth.: ORS 468 & ORS 468A
Stats. Implemented: ORS 468A.025
Hist.: DEQ 37, f. 2-15-72, ef. 3-1-72; DEQ 3-1982, f. & ef. 1-29-82; DEQ 4-1993, f. & cert. ef. 3-10-93; DEQ 14-1999, f. & cert. ef. 10-14-99, Renumbered from 340-022-0020

340-228-0130

Exemptions

Exempted from the requirements of OAR 340-228-0100 through 340-228-0120 are:

(1) Fuels used exclusively for the propulsion and auxiliary power requirements of vessels, railroad locomotives, and diesel motor vehicles.

(2) With prior approval of DEQ, fuels used in such a manner or control provided such that sulfur dioxide emissions can be demonstrated to be equal to or less than those resulting from the combustion of fuels complying with the limitations of OAR 340-228-0100 through 340-228-0120.

[NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the Environmental Quality Commission under OAR 340-200-0040.]

Stat. Auth.: ORS 468 & ORS 468A
Stats. Implemented: ORS 468A.025
Hist.: DEQ 37, f. 2-15-72, ef. 3-1-72; DEQ 4-1993, f. & cert. ef. 3-10-93; DEQ 14-1999, f. & cert. ef. 10-14-99, Renumbered from 340-022-0025

General Emission Standards for Fuel Burning Equipment

340-228-0200

Sulfur Dioxide Standards

The following emission standards are only applicable to sources installed, constructed, or modified after January 1, 1972 except recovery furnaces regulated in division 234:

(1) For fuel burning equipment having a heat input capacity between 150 million BTU per hour and 250 million BTU, no person may cause, suffer, allow, or permit the emission into the atmosphere of sulfur dioxide in excess of:

(a) 1.4 lb. per million BTU heat input, maximum three-hour average, when liquid fuel is burned;

(b) 1.6 lb. per million BTU heat input, maximum three-hour average, when solid fuel is burned.

(2) For fuel burning equipment having a heat input capacity of more than 250 million BTU per hour, no person may cause, suffer, allow, or permit the emission into the atmosphere of sulfur dioxide in excess of:

(a) 0.8 lb. per million BTU heat input, maximum three-hour average, when liquid fuel is burned;

(b) 1.2 lb. per million BTU heat input, maximum three-hour average, when solid fuel is burned.

NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the Environmental Quality Commission under OAR 340-200-0040.

Stat. Auth.: ORS 468 & 468A
Stats. Implemented: ORS 468.020 & 468A.025
Hist.: DEQ 37, f. 2-15-72, ef. 3-1-72; DEQ 4-1993, f. & cert. ef. 3-10-93; DEQ 22-1996, f. & cert. ef. 10-22-96; DEQ 14-1999, f. & cert. ef. 10-14-99, Renumbered from 340-022-0055; DEQ 8-2007, f. & cert. ef. 11-8-07; DEQ 7-2011, f. & cert. ef. 6-24-11; Administrative correction, 2-6-12; DEQ 1-2012, f. & cert. ef. 5-17-12

340-228-0210

Grain Loading Standards

(1) No person may cause, suffer, allow, or permit particulate matter emission from any fuel burning equipment in excess of:

(a) For sources installed, constructed, or modified before June 1, 1970:

(A) 0.2 grains per dry standard cubic foot through March 31, 2015;

(B) 0.20 grains per dry standard cubic foot from April 1, 2015 through March 31, 2019.

(b) For sources installed, constructed, or modified on or after June 1, 1970:

(A) 0.1 grains per dry standard cubic foot through March 31, 2019 if located more than 5 miles of a PM10/PM2.5 sustainment area, nonattainment area, reattainment area, or maintenance area;

(B) 0.1 grains per dry standard cubic foot through March 31, 2015 if located within 5 miles of a PM10/PM2.5 sustainment area, nonattainment area, reattainment area, or maintenance area;

(C) 0.10 grains per dry standard cubic foot after March 31, 2015 if located within 5 miles of a PM10/PM2.5 sustainment area, nonattainment area, reattainment area, or maintenance area;

(c) For sources installed, constructed or modified after March 31, 2014, 0.10 grains per dry standard cubic foot.

(d) For all sources, 0.10 grains per dry standard cubic foot after March 31, 2019.

(e) The owner or operator of an source installed, constructed or modified before April 1, 2014 who is unable to comply with any of the compliance dates specified in paragraphs (a)(B), (b)(C), and (d) above may request that DEQ grant an extension allowing the source up to one additional year to comply with the standard, provided that the owner or operator submits an engineering report signed by a registered professional engineer that demonstrates that the source cannot comply with the standard without making significant changes to the equipment or control equipment or adding control equipment. The request for an extension must be submitted no later than 90 days prior to the compliance dates.

(2) Compliance with the emissions standards in section (1) is determined using Oregon Method 5, or an alternative method approved by DEQ.

(a) For external combustion devices that burn wood fuel by itself or in combination with any other fuel, the emission results are corrected to 12% CO2.

(b) For external combustion devices that burn fuels other than wood, the emission results are corrected to 50% excess air.

(3) This rule does not apply to solid fuel burning devices that have been certified under OAR 340-262-0500.

NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the Environmental Quality Commission under OAR 340-200-0040.

Stat. Auth.: ORS 468 & 468A
Stats. Implemented: ORS 468.020 & 468A.025
Hist.: DEQ 16, f. 6-12-70, ef. 7-11-70; DEQ 12-1979, f. & ef. 6-8-79; DEQ 6-1981, f. & ef. 2-17-81; DEQ 18-1982, f. & ef. 9-1-82; DEQ 4-1993, f. & cert. ef. 3-10-93; DEQ 3-1996, f. & cert. ef. 1-29-96; DEQ 14-1999, f. & cert. ef. 10-14-99, Renumbered from 340-021-0020; DEQ 8-2007, f. & cert. ef. 11-8-07; DEQ 7-2011, f. & cert. ef. 6-24-11; Administrative correction, 2-6-12; DEQ 1-2012, f. & cert. ef. 5-17-12

Federal Acid Rain Program

340-228-0300

Federal Regulations Adopted by Reference

(1) 40 CFR Parts 72, 75, and 76 are by this reference adopted and incorporated herein, for purposes of implementing an acid rain program that meets the requirements of title IV of the Clean Air Act. The term "permitting authority" means the Oregon Department of Environmental Quality and the term "Administrator" shall mean the Administrator of the United States Environmental Protection Agency.

(2) If the provisions or requirements of 40 CFR Part 72 conflict with or are not included in OAR 340 divisions 218 or 220, the Part 72 provisions and requirements shall apply and take precedence.

[Publications: Publications referenced are available from the agency.]

Stat. Auth.: ORS 468.020 & 468.310(2)
Stats. Implemented: ORS 468A.025
Hist.: DEQ 32-1994, f. & cert. ef. 12-22-94; DEQ 14-1999, f. & cert. ef. 10-14-99, Renumbered from 340-022-0075; DEQ 22-2000, f. & cert. ef. 12-18-00; DEQ 13-2006, f. & cert. ef. 12-22-06; DEQ 5-2011, f. 4-29-11, cert. ef. 5-1-11

340-228-0400

Purpose

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0410

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0420

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0430

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0440

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0450

Registration

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0460

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0470

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0480

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0490

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0500

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0510

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0520

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

340-228-0530

Integration into Permits

Repealed

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03

Repealed

[NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the EQC under OAR 340-200-0040.]

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.035
Hist.: DEQ 19-2003, f. & cert. ef. 12-12-03