DIVISION 222

STATIONARY SOURCE PLANT SITE EMISSION LIMITS

340-222-0010

Policy

The Commission recognizes the need to establish a more definitive method for regulating increases and decreases in air emissions of permit holders. However, except as needed to protect ambient air quality standards, prevention of significant deterioration increments and visibility, the Commission does not intend to: limit the use of existing production capacity of any air quality permittee; cause any undue hardship or expense to any permittee who wishes to use existing unused productive capacity; or create inequity within any class of permittees subject to specific industrial standards that are based on emissions related to production.

[NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the EQC under OAR 340-200-0040.]

Stat. Auth.: ORS 468 & ORS 468A
Stats. Implemented: ORS 468 & ORS 468A
Hist.: DEQ 25-1981, f. & ef. 9-8-81; DEQ 4-1993, f. & cert. ef. 3-10-93; DEQ 12-1993, f. & cert. ef. 9-24-93; Renumbered from 340-020-0300; DEQ 19-1993, f. & cert. ef. 11-4-93; DEQ 14-1999, f. & cert. ef. 10-14-99, Renumbered from 340-028-1000; DEQ 6-2001, f. 6-18-01, cert. ef. 7-1-01

340-222-0020

Applicability

(1) Plant Site Emission Limits (PSELs) will be included in all Air Contaminant Discharge Permits (ACDP) and Oregon Title V Operating Permits, except as provided in section (3), as a means of managing airshed capacity by regulating increases and decreases in air emissions. Except as provided in OAR 340-222-0060 or 340-222-0070, all ACDP and Title V sources are subject to PSELs for all regulated pollutants. The Department will incorporate PSELs into permits when issuing a new permit or renewing or modifying an existing permit.

(2) The emissions limits established by PSELs provide the basis for:

(a) Assuring reasonable further progress toward attaining compliance with ambient air standards;

(b) Assuring compliance with ambient air standards and Prevention of Significant Deterioration increments;

(c) Administering offset and banking programs; and

(d) Establishing the baseline for tracking the consumption of Prevention of Significant Deterioration Increments.

(3) PSELs are not required for:

(a) Pollutants that will be emitted at less than the de minimis emission level listed in OAR 340-200-0020 from the entire source,

(b) Short Term Activity and Basic ACDPs; or

(c) Hazardous air pollutants as listed in OAR 340-244-0040 Table 1; Early Reduction High Risk Pollutants listed in 340-244-0120 Table 2; or Accidental Release Substances listed in 340-244-0230 Table 3.

(4) Generic PSELs may be used for any category of ACDP or Title V permit.

NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the EQC under OAR 340-200-0040.

[ED. NOTE: Tables referenced are available from the agency.]

Stat. Auth.: ORS 468.020 & 468A.040
Stats. Implemented: ORS 468.020, 468.065 & 468A.025
Hist.: DEQ 25-1981, f. & ef. 9-8-81; DEQ 4-1993, f. & cert. ef. 3-10-93; DEQ 12-1993, f. & cert. ef. 9-24-93, Renumbered from 340-020-0301; DEQ 19-1993, f. & cert. ef. 11-4-93; DEQ 22-1995, f. & cert. ef. 10-6-95; DEQ 22-1996, f. & cert. ef. 10-22-96; DEQ 14-1998, f. & cert. ef. 9-14-98; DEQ 14-1999, f. & cert. ef. 10-14-99, Renumbered from 340-028-1010; DEQ 6-2001, f. 6-18-01, cert. ef. 7-1-01; DEQ 4-2008(Temp). f. 3-4-08, cert. ef. 3-6-08 thru 9-1-08; DEQ 11-2008, f. & cert. ef. 8-29-08

340-222-0030

Definitions

The definitions in OAR 340-200-0020 and this rule apply to this division. If the same term is defined in this rule and 340-200-0020, the definition in this rule applies to this division.

[NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the Environmental Quality Commission under OAR 340-200-0040.]

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.025
Hist.: DEQ 14-1999, f. & cert. ef. 10-14-99

Criteria for Establishing Plant Site Emission Limits

340-222-0040

Generic Annual PSEL

(1) Sources with capacity less than the Significant Emission Rate (SER) will receive a Generic PSEL unless they have a netting basis and request a source specific PSEL under 340-222-0041.

(2) A Generic PSEL may be used for any pollutant that will be emitted at less than the SER. The netting basis for a source with a generic PSEL is zero.

[NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the EQC under OAR 340-200-0040.]

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A
Hist.: DEQ 25-1981, f. & ef. 9-8-81; DEQ 4-1993, f. & cert. ef. 3-10-93; DEQ 12-1993, f. & cert. ef. 9-24-93; Renumbered from 340-020-0310; DEQ 19-1993, f. & cert. ef. 11-4-93; DEQ 22-1995, f. & cert. ef. 10-6-95; DEQ 14-1999, f. & cert. ef. 10-14-99, Renumbered from 340-028-1020; DEQ 6-2001, f. 6-18-01, cert. ef. 7-1-01

340-222-0041

Source Specific Annual PSEL

(1) For sources with potential to emit less than the SER, that request a source specific PSEL, an initial source specific PSEL will be set equal to the Generic PSEL.

(2) For sources with potential to emit greater than or equal to the SER, an initial source specific PSEL will be set equal to the source's potential to emit or netting basis, whichever is less.

(3) If an applicant wants an annual PSEL at a rate greater than the netting basis, the applicant must:

(a) Demonstrate that the requested increase over the netting basis is less than the SER; or

(b) For increases equal to or greater than the SER over the netting basis, but not subject to New Source Review (OAR 340 division 224):

(A) If located within, or creating a significant air quality impact as defined in OAR 340-200-0020 upon, an area designated as nonattainment in OAR 340-204-0030, the applicant must obtain offsets and demonstrate a net air quality benefit in accordance with 340-225-0090.

(B) If located within, or creating a significant air quality impact as defined in OAR 340-200-0020 upon, an area designated as maintenance in 340-204-0040, the applicant must

(i) Obtain offsets and demonstrate a net air quality benefit in accordance with OAR 340-225-0090;

(ii) Obtain an allocation from an available growth allowance in accordance with the applicable maintenance plan; or

(iii) Demonstrate compliance with the air quality impact levels in OAR 340-224-0060(2)(c) or (2)(d), whichever applies to the maintenance area, by conducting an air quality analysis in accordance with 340-225-0045.

(C) If located within an attainment, maintenance, or unclassifiable area, the applicant must demonstrate compliance with the NAAQS and PSD increments by conducting an air quality analysis in accordance with OAR 340-225-0050(1) and (2) and 340-225-0060.

(D) For federal major sources, the applicant must demonstrate compliance with AQRV protection in accordance with OAR 340-225-0050(3) and 340-225-0070.

(c) For increases equal to or greater than the SER over the netting basis and subject to New Source Review, the applicant must demonstrate that the applicable New Source Review requirements have been satisfied.

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A
Hist.: DEQ 6-2001, f. 6-18-01, cert. ef. 7-1-01; DEQ 11-2002, f. & cert. ef. 10-8-02

340-222-0042

Short Term PSEL

(1) For sources located in areas with established short term SER (OAR 340-200-0020 Table 3), PSELs are required on a short term basis for those pollutants that have a short term SER. The short term averaging period is daily, unless emissions cannot be monitored on a daily basis. The averaging period for short term PSELs can never be greater than monthly.

(a) For existing sources, the initial short term PSEL will be set as:

(A) the lesser of the short term capacity or the current permit's short term PSEL, if each is greater than or equal to the short term SER; or

(B) the generic PSEL, if either the short term capacity or the current short term PSEL is less than the short term SER.

(b) For new sources, the initial short term PSEL will be zero.

(2) If an applicant wants a short term PSEL at a rate greater than the initial short term PSEL, the applicant must:

(a) Demonstrate that the requested increase over the initial short term PSEL is less than the significant emission rate (Note: In this case new sources would get a generic PSEL); or

(b) For increases equal to or greater than the SER over the initial short term PSEL:

(A) Obtain offsets and demonstrate a net air quality benefit in accordance with OAR 340-225-0090;

(B) Obtain an allocation from an available growth allowance in accordance with the applicable maintenance plan; or

(C) For carbon monoxide, demonstrate that the source or modification will not cause or contribute to an air quality impact equal to or greater than 0.5 mg/m3 (8 hour average) and 2 mg/m3 (1 hour average).

(D) For federal major sources, demonstrate compliance with air quality related values (AQRV) protection in accordance with OAR 340-225-0070.

(3) Once the short term PSEL is increased pursuant to section (2) of this rule, the increased level becomes the initial short term PSEL for future evaluations.

[ED. NOTE: Tables referenced are available from the agency.]

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A
Hist.: DEQ 6-2001, f. 6-18-01, cert. ef. 7-1-01; DEQ 5-2011, f. 4-29-11, cert. ef. 5-1-11

340-222-0043

General Requirements for All PSEL

(1) No PSEL may allow emissions in excess of those allowed by any applicable federal or state regulation or by any specific permit conditions unless the source meets the specific provisions of OAR 340-226-0400 (Alternative Emission Controls).

(2) Source specific PSELs may be changed pursuant to the Department's rules for permit modifications when:

(a) Errors are found or better data is available for calculating PSELs

(b) More stringent control is required by a rule adopted by the Commission; or

(c) The Department modifies a permit pursuant to OAR 340-216-0084, Modification of a Permit, or 340-218-0200, Reopenings.

(3) Annual PSELs are established on a rolling 12 consecutive month basis and will limit the source's potential to emit.

(4) In order to maintain the netting basis, permittees must maintain either a Standard ACDP or an Oregon Title V Operating Permit. A request by a permitee to be assigned any other type of an ACDP sets the netting basis at zero upon issuance of the other type of permit.

[NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the EQC under OAR 340-200-0040.]

Stat. Auth.: ORS 468 & ORS 468A
Stats. Implemented: ORS 468 & ORS 468A
Hist.: DEQ 6-2001, f. 6-18-01, cert. ef. 7-1-01

340-222-0045

Unassigned Emissions

(1) Purpose. The purpose of unassigned emissions is to track and manage the difference in the quantity of emissions between the netting basis and what the source could emit based on the facility's current physical and operational design.

(2) Establishing unassigned emissions.

(a) Unassigned emissions equal the netting basis minus the source's current PTE, minus any banked emission reduction credits. Unassigned emissions are zero if this result is negative.

(b) Unused capacity created after the effective date of this rule due to reduced potential to emit that is not banked or expired emission reduction credits (OAR 340-268-0030), increase unassigned emissions on a ton for ton basis.

(3) Maximum unassigned emissions.

(a) Except as provided in paragraph (c) of this section, unassigned emissions will be reduced to not more than the SER (OAR 340-200-0020 Table 2) on July 1, 2007 and at each permit renewal following this date.

(b) The netting basis is reduced by the amount that unassigned emissions are reduced.

(c) In an AQMA where the EPA requires an attainment demonstration based on dispersion modeling, unassigned emissions are not subject to reduction under this rule.

(4) Using unassigned emissions.

(a) Unassigned emissions may be used for internal netting to allow an emission increase at the existing source in accordance with the permit.

(b) Unassigned emissions may not be banked or transferred to another source.

(c) Emissions that are removed from the netting basis are unavailable for netting in any future permit actions.

(5) Upon renewal, modification or other reopening of a permit after July 1, 2002 the unassigned emissions will be established with an expiration date of July 1, 2007 for all unassigned emissions in excess of the SER. Each time the permit is renewed after July 1, 2007 the unassigned emissions will be established again and reduced upon the following permit renewal to no more than the SER for each pollutant in OAR 340-200-0020 Table 2.

NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the EQC under OAR 340-200-0040.

[ED. NOTE: Tables referenced are available from the agency.]

Stat. Auth.: ORS 468.020 & 468A.310
Stats. Implemented: ORS 468 & 468A
Hist.: DEQ 6-2001, f. 6-18-01, cert. ef. 7-1-01; DEQ 5-2011, f. 4-29-11, cert. ef. 5-1-11

340-222-0060

Plant Site Emission Limits for Sources of Hazardous Air Pollutants

(1) The Department may establish PSELs for hazardous air pollutants (HAPs) if an owner or operator:

(a) Elects to establish a PSEL for combined HAPs emitted for purposes of determining emission fees as prescribed in OAR 340 division 220; or

(b) Asks the Department to create an enforceable PTE limit.

(2) PSELs will be set only for individual or combined HAPs and will not list HAPs by name. The PSEL will be set on a rolling 12 month basis and will be either:

(a) The generic PSEL if the permittee proposes a limit less than that level; or

(b) The level the permittee establishes necessary for the source if greater than the generic PSEL.

(3) The Alternative Emissions Controls (Bubble) provisions of OAR 340-226-0400 do not apply to emissions of HAPs.

[NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the EQC under OAR 340-200-0040.]

Stat. Auth.: ORS 468.020
Stats. Implemented: ORS 468A.025
Hist.: DEQ 12-1993, f. & cert. ef. 9-24-93; DEQ 22-1995, f. & cert. ef. 10-6-95; DEQ 19-1996, f. & cert. ef. 9-24-96; DEQ 14-1999, f. & cert. ef. 10-14-99, Renumbered from 340-028-1050; DEQ 6-2001, f. 6-18-01, cert. ef. 7-1-01

340-222-0070

Plant Site Emission Limits for Insignificant Activities

(1) For purposes of establishing PSELs, emissions from categorically insignificant activities listed in OAR 340-200-0020 are not considered under 340-222-0020, except as provided in section (3) of this rule.

(2) For purposes of establishing PSELs, emissions from aggregate insignificant emissions listed in OAR 340-200-0020 are considered under 340-222-0020.

(3) For purposes of determining New Source Review or Prevention of Significant Deterioration applicability under OAR 340 division 224, emissions from insignificant activities are considered.

[NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the EQC under OAR 340-200-0040.]

Stat. Auth.: ORS 468 & ORS 468A
Stats. Implemented: ORS 468.020, ORS 468A.025, ORS 468A.040, & ORS 468A.045.
Hist.: DEQ 12-1993, f. & cert. ef. 9-24-93; DEQ 19-1993, f. & cert. ef. 11-4-93; DEQ 2-1996, f. & cert. ef. 1-29-96; DEQ 14-1999, f. & cert. ef. 10-14-99, Renumbered from 340-028-1060; DEQ 6-2001, f. 6-18-01, cert. ef. 7-1-01

340-222-0080

Plant Site Emission Limit Compliance

(1) The permittee must monitor pollutant emissions or other parameters that are sufficient to produce the records necessary for demonstrating compliance with the PSEL.

(2) The frequency of the monitoring and associated averaging periods must be as short as possible and consistent with that used in the compliance method.

(3)(a) For annual PSELs, the permittee must monitor appropriate parameters and maintain all records necessary for demonstrating compliance with the annual PSEL at least monthly and be able to determine emissions on a rolling 12 consecutive month basis.

(b) For short term PSELs, the permittee must monitor appropriate parameters and maintain all records necessary for demonstrating compliance with any short term PSEL at least as frequently as the short term PSEL averaging period.

(4) The applicant must specify in the permit application the method(s) for determining compliance with the PSEL. The Department will review the method(s) and approve or modify, as necessary, to assure compliance with the PSEL. The Department will include PSEL compliance monitoring methods in all permits that contain PSELs.

(5) Depending on source operations, one or more of the following methods may be acceptable:

(a) Continuous emissions monitors;

(b) Material balance calculations;

(c) Emissions calculations using approved emission factors and process information;

(d) Alternative production or process limits; and

(e) Other methods approved by the Department.

(6) When annual reports are required, the permittee must include the emissions total for each consecutive 12 month period during the calendar year, unless otherwise specified by a permit condition.

[NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the EQC under OAR 340-200-0040.]

Stat. Auth.: ORS 468 & ORS 468A
Stats. Implemented: ORS 468 & ORS 468A
Hist.: DEQ 6-2001, f. 6-18-01, cert. ef. 7-1-01

340-222-0090

Combining and Splitting Sources

(1) When two or more sources combine into one source:

(a) The sum of the netting basis for all the sources is the combined source netting basis.

(b) The combined source is regulated as one source, except:

(A) the simple act of combining sources, without an increase over the combined PSEL, does not subject the combined source to New Source Review.

(B) if the combined source PSEL, without a requested increase over the existing combined PSEL, exceeds the combined netting basis plus the SER, the source may continue operating at the existing combined source PSEL without becoming subject to New Source Review until an increase in the PSEL is requested or the source is modified. If an increase in the PSEL is requested or the source is modified, the Department will evaluate whether New Source Review applies.

(2) When one source is split into two or more separate sources:

(a) The netting basis and the SER for the original source is split amongst the new sources as requested by the original permittee.

(b) The split of netting basis and SER must either:

(A) be sufficient to avoid New Source Review for each of the newly created sources or

(B) the newly created source(s) that become subject to New Source Review must comply with the requirements of OAR 340 division 224 before beginning operation under the new arrangement.

(3) The owner of the device or emissions unit must maintain records of physical changes and changes in operation occurring since the baseline period.

[NOTE: This rule is included in the State of Oregon Clean Air Act Implementation Plan as adopted by the EQC under OAR 340-200-0040.]

Stat. Auth.: ORS 468 & ORS 468A
Stats. Implemented: ORS 468 & ORS 468A
Hist.: DEQ 6-2001, f. 6-18-01, cert. ef. 7-1-01