# DIVISION 200

# GENERAL AIR POLLUTION PROCEDURES AND DEFINITIONS

# 340-200-8005

# Table 1 Significant Impact Levels

# As used in divisions 200, 209, and 225, Table 1 Significant Impact Levels refers to the table below.

| # Table 1 # Significant Impact Levels |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| # Pollutant | # Averaging Time | # Air Quality Area Designation |  |  |  |
|  |  | # Class I | # Class II | # Class III |  |
| # SO2 (µg/m3) | # Annual | # 0.10 | # 1.0 | # 1.0 |  |
|  | # 24-hour | # 0.20 | # 5.9 | # 5.0 |  |
|  | # 3-hour | # 1.0 | # 25.0 | # 25.0 |  |
| # PM10 (µg/m3) | # Annual | # 0.20 | # 0.20 | # 0.20 |  |
|  | # 24-hour | # 0.30 | # 1.0 | # 1.0 |  |
| # PM2.5 (µg/m3) | # Annual | # 0.06 | # 0.3 | # 0.3 |  |
|  | # 24-hour | # 0.07 | # 1.2 | # 1.2 |  |
| # NO2 (µg/m3) | # Annual | # 0.10 | # 1.0 | # 1.0 |  |
| # CO (mg/m3) | # 8-hour | # --- | # 0.5 | # 0.5 |  |
|  | # 1-hour | # --- | # 2.0 | # 2.0 |  |

[ED. NOTE: This rule amended and renumbered from OAR 340-200-0020.]

Micrograms/cubic meter

Millligrams/cubic meter