| # Table 1 # Significant Impact Levels # OAR 340-200-8000 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| # Pollutant | # Averaging Time | # Air Quality Area Designation |  |  |  |
|  |  | # Class I | # Class II | # Class III |  |
| # SO2 (µg/m3) | # Annual | # 0.10 | # 1.0 | # 1.0 |  |
|  | # 24-hour | # 0.20 | # 5.9 | # 5.0 |  |
|  | # 3-hour | # 1.0 | # 25.0 | # 25.0 |  |
| # PM10 (µg/m3) | # Annual | # 0.20 | # 0.20 | # 0.20 |  |
|  | # 24-hour | # 0.30 | # 1.0 | # 1.0 |  |
| # PM2.5 (µg/m3) | # Annual | # 0.06 | # 0.3 | # 0.3 |  |
|  | # 24-hour | # 0.07 | # 1.2 | # 1.2 |  |
| # NO2 (µg/m3) | # Annual | # 0.10 | # 1.0 | # 1.0 |  |
| # CO (mg/m3) | # 8-hour | # --- | # 0.5 | # 0.5 |  |
|  | # 1-hour | # --- | # 2.0 | # 2.0 |  |

[ED. NOTE: This rule amended and renumbered from OAR 340-200-0020.]

| # Table # 2 # Significant Emission Rates # (OAR 340-200-8010) |  |  |
| --- | --- | --- |
| # Pollutant | # Emission Rate |  |
| # Greenhouse Gases (CO2e) | # 75,000 tons/year |  |
| # Carbon Monoxide | 100 tons/year |  |
| # Nitrogen Oxides (NOX) | 40 tons/year |  |
| # Particulate Matter | 25 tons/year |  |
| # PM10 | 15 tons/year |  |
| # Direct PM2.5 | 10 tons/year |  |
| # PM2.5 precursors (SO2 or NOx) | 40 tons/year |  |
| # Sulfur Dioxide (SO2) | 40 tons/year |  |
| # Volatile Organic Compounds (VOC) | 40 tons/year |  |
| # Ozone precursors (VOC or NOx) | 40 tons/year |  |
| # Lead | 0.6 ton/year |  |
| # Fluorides | # 3 tons/year |  |
| # Sulfuric Acid Mist | # 7 tons/year |  |
| # Hydrogen Sulfide | # 10 tons/year |  |
| # Total Reduced Sulfur (including hydrogen sulfide) | # 10 tons/year |  |
| # Reduced sulfur compounds (including hydrogen sulfide) | # 10 tons/year |  |
| # Municipal waste combustor organics (measured as total tetra- through octa- chlorinated dibenzo-p-dioxins and dibenzofurans) | # 0.0000035 ton/year |  |
| # Municipal waste combustor metals (measured as particulate matter) | # 15 tons/year |  |
| # Municipal waste combustor acid gases (measured as sulfur dioxide and hydrogen chloride) | # 40 tons/year |  |
| # Municipal solid waste landfill emissions (measured as nonmethane organic compounds) | # 50 tons/year |  |

# [ED. NOTE: This rule amended and renumbered from OAR 340-200-0020.]

| # Table # 3 # Significant Emission Rates for the Medford-Ashland Air Quality Maintenance Area # (OAR 340-200-8020) |  |  |  |
| --- | --- | --- | --- |
| # Air Contaminant | # Emission Rate |  |  |
|  | Annual | Day |  |
| # PM10 | 5.0 tons/year | # 50.0 pounds |  |

# [ED. NOTE: This rule amended and renumbered from OAR 340-200-0020.]

| # Table # 4 # De Minimis Emission Levels # (OAR 340-200-8030) |  |  |
| --- | --- | --- |
| # Pollutant | # De minimis (tons/year, except as noted) |  |
| # Greenhouse Gases (CO2e) | # 2,756 |  |
| # CO | # 1 |  |
| # NOx | # 1 |  |
| # SO2 | # 1 |  |
| # VOC | # 1 |  |
| # PM | # 1 |  |
| # PM10 (except Medford AQMA) | # 1 |  |
| # PM10/PM2.5 (Medford AQMA) | # 0.5 [5.0 lbs/day] |  |
| # Direct PM2.5 | # 1 |  |
| # Lead | # 0.1 |  |
| # Fluorides | # 0.3 |  |
| # Sulfuric Acid Mist | # 0.7 |  |
| # Hydrogen Sulfide | # 1 |  |
| # Total Reduced Sulfur (including hydrogen sulfide) | # 1 |  |
| # Reduced Sulfur | # 1 |  |
| # Municipal waste combustor organics (Dioxin and furans) | # 0.0000005 |  |
| # Municipal waste combustor metals | # 1 |  |
| # Municipal waste combustor acid gases | # 1 |  |
| # Municipal solid waste landfill gases | # 1 |  |
| # Single HAP | # 1 |  |
| # Combined HAP (aggregate) | # 1 |  |

# [ED. NOTE: This rule amended and renumbered from OAR 340-200-0020.]

| # Table # 5 # Generic PSELs # (OAR 340-200-8040) |  |  |
| --- | --- | --- |
| # Pollutant | # Generic PSEL # (tons/year, except as noted) |  |
| # Greenhouse Gases (CO2e) | # 74,000 |  |
| # CO | # 99 |  |
| # NOx | # 39 |  |
| # SO2 | # 39 |  |
| # VOC | # 39 |  |
| # PM | # 24 |  |
| # PM10 (except Medford AQMA) | # 14 |  |
| # PM10/PM2.5 (Medford AQMA) | # 4.5 [49 lbs/day] |  |
| # PM2.5 | # 9 |  |
| # Lead | # 0.5 |  |
| # Fluorides | # 2 |  |
| # Sulfuric Acid Mist | # 6 |  |
| # Hydrogen Sulfide | # 9 |  |
| # Total Reduced Sulfur (including hydrogen sulfide) | # 9 |  |
| # Reduced Sulfur | # 9 |  |
| # Municipal waste combustor organics (Dioxin and furans) | # 0.0000030 |  |
| # Municipal waste combustor metals | # 14 |  |
| # Municipal waste combustor acid gases | # 39 |  |
| # Municipal solid waste landfill gases | # 49 |  |
| # Single HAP | # 9 |  |
| # Combined HAPs (aggregate) | # 24 |  |

# [ED. NOTE: This rule amended and renumbered from OAR 340-200-0020(56).]

Micrograms/cubic meter

Millligrams/cubic meter