Comparison of Current and Proposed Requirements for PSEL increases above SER in Attainment, Nonattainment and Maintenance Areas (Minor and Major NSR/PSD)

Nonattainment Areas

| Program Elements | Current Requirements | Proposed Requirements |
| --- | --- | --- |
|  | Source Emissions after the PSEL increase 100 tons per year or more |  |
| Source Classification | Major | Federal Major |
| Preconstruction Monitoring | n/a | n/a |
| Control Technology | LAER * | LAER * |
| AQ Analysis-NAAQS/Increments/ AQRV | AQRV analysis (Federal Majors only) | AQRV analysis |
| NAQB | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants ** Reduce impacts at majority of receptors; and Impacts less than SIL at all receptor | Offsets - 1.1:1 for ozone - 1.2:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources Source plus competing sources since area was designated less than 10% of the NAAQS |
| Additional Impacts | n/a | n/a |
| Alternatives Analysis | Alternative sites analysis * | Alternative sites analysis * |
| Certify Compliance | Certify compliance for all sources * | Certify compliance for all sources * |
|  | Source Emissions after the PSEL increase SER to 99 tons per year |  |
| Source Classification | Major | Minor |
| Preconstruction Monitoring | n/a | n/a |
| Control Technology | LAER * | BACT * |
| AQ Analysis-NAAQS/Increments/ AQRV | AQRV analysis | n/a |
| NAQB | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants ** - Reduce impacts at majority of receptors; and - Impacts less than SIL at all receptors | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources - Source plus competing sources since area was designated less than 10% of the NAAQS |
| Additional Impacts | n/a | n/a |
| Alternatives Analysis | Alternative sites analysis * | n/a |
| Certify Compliance | Certify compliance for all sources * | n/a |
|  | Source Emissions after the PSEL increase less than SER |  |
| Source Classification | Minor | Minor |
|  | NC rules or ACDP | NC rules or ACDP, no change |

*  If a Major Modification is involved

** Offset ratio varies for certain areas such as Medford PM10, etc.

Somewhere explain change to FM classification

Maintenance Areas

| Program Elements | Current Requirements | Proposed Requirements |
| --- | --- | --- |
|  | Source Emissions after the PSEL increase 100 tons per year or more |  |
| Source Classification | Major | Federal Major |
| Preconstruction Monitoring | Preconstruction Monitoring *** | Preconstruction Monitoring *** |
| Control Technology | BACT * | BACT * |
| AQ Analysis-NAAQS/Increments/ AQRV | NAAQS and PSD Increments AQRV analysis (Federal Majors) | NAAQS and PSD Increments AQRV analysis |
| NAQB | - Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants ** and NAQB - Reduce impacts at majority of receptors; and - Impacts less than SIL at all receptors Or - Growth allowance Or - Model below maintenance area limits | - Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources ** and NAQB - Source plus competing sources since area was designated less than 10% of the NAAQS Or - Growth allowance Or - Model below maintenance area limits |
| Additional Impacts | Additional impacts analysis (Federal Majors) | Additional impacts analysis |
| Alternatives Analysis | n/a | n/a |
| Certify Compliance | n/a | n/a |
|  | Source Emissions after the PSEL increase SER to 99 tons per year |  |
| Source Classification | Major | Minor |
| Preconstruction Monitoring | Preconstruction Monitoring *** |  |
| Control Technology | BACT * | BACT * |
| AQ Analysis-NAAQS/Increments/ AQRV | NAAQS and PSD Increments AQRV analysis (Federal Majors) | NAAQS and PSD Increments |
| NAQB | - Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants ** and NAQB - Reduce impacts at majority of receptors; and - Impacts less than SIL at all receptors Or - Growth allowance Or - Model below maintenance area limits | - Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants ** and NAQB - Reduce impacts at majority of receptors; and - Impacts less than SIL at all receptors Or - Growth allowance Or - Model below maintenance area limits |
| Additional Impacts | Additional impacts analysis (Federal Majors) |  |
| Alternatives Analysis | n/a | n/a |
| Certify Compliance | n/a | n/a |
|  | Source Emissions after the PSEL increase less than SER |  |
| Source Classification | Minor | Minor |
|  | NC rules or ACDP | NC rules or ACDP, no change |

*  If a Major Modification is involved

** Offset ratio varies for certain areas such as Medford PM10, etc.

*** If impacts are greater than the Significant Monitoring Concentration

Attainment Areas

| Program Elements | Current Requirements | Proposed Requirements |
| --- | --- | --- |
|  | Source Emissions after the PSEL increase 100/250 tons per year or more |  |
| Source Classification | Federal Major | Federal Major |
| Preconstruction Monitoring | Preconstruction monitoring *** | Preconstruction monitoring *** |
| Control Technology | BACT * | BACT * |
| AQ Analysis-NAAQS/Increments/ AQRV | NAAQS and PSD Increments AQRV Analysis | NAAQS and PSD Increments AQRV Analysis |
| NAQB | n/a | n/a |
| Additional Impacts | Additional impacts analysis * | Additional impacts analysis * |
| Alternatives Analysis | n/a | n/a |
| Certify Compliance | n/a | n/a |
|  | Source Emissions after the PSEL increase SER to 99/249 tons per year |  |
| Source Classification | Minor | Minor |
| Preconstruction Monitoring | n/a | n/a |
| Control Technology | n/a | n/a |
| AQ Analysis-NAAQS/Increments/ AQRV | NAAQS and PSD Increments | NAAQS and PSD Increments |
| NAQB | n/a | n/a |
| Additional Impacts | n/a | n/a |
| Alternatives Analysis | n/a | n/a |
| Certify Compliance | n/a | n/a |
|  | Source Emissions after the PSEL increase less than SER |  |
| Source Classification | Minor | Minor |
|  | NC rules or ACDP | NC rules or ACDP, no change |

*  If a Major Modification is involved

** Offset ratio varies for certain areas such as Medford PM10, etc.

*** If impacts are greater than the Significant Monitoring Concentration

Sustainment Comparison to Attainment and Nonattainment

| Program Elements | Attainment | Sustainment | Nonattainment |
| --- | --- | --- | --- |
|  | Source Emissions after the PSEL increase 100/250 tons per year or more |  |  |
| Source Classification | Federal Major | Federal Major | Federal Major |
| Preconstruction Monitoring | Preconstruction monitoring *** | Preconstruction monitoring (required) | n/a |
| Control Technology | BACT * | BACT * | LAER * |
| AQ Analysis-NAAQS/Increments/ AQRV | NAAQS and PSD Increments AQRV Analysis | NAAQS and PSD Increments AQRV Analysis | AQRV analysis |
| NAQB | n/a | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources | Offsets - 1.1:1 for ozone - 1.2:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources Source plus competing sources since area was designated less than 10% of the NAAQS |
| Additional Impacts | Additional impacts analysis * | Additional impacts analysis * | n/a |
| Alternatives Analysis | n/a | n/a | Alternative sites analysis * |
| Certify Compliance | n/a | n/a | Certify compliance for all sources * |
|  | Source Emissions after the PSEL increase 100 to 99/249 tons per year |  |  |
| Source Classification | Minor | Federal Major | Federal Major |
| Preconstruction Monitoring | n/a | Preconstruction monitoring (required) | n/a |
| Control Technology | n/a | BACT * | LAER * |
| AQ Analysis-NAAQS/Increments/ AQRV | NAAQS and PSD Increments | NAAQS and PSD Increments AQRV Analysis | AQRV analysis |
| NAQB | n/a | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources | Offsets - 1.1:1 for ozone - 1.2:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources Source plus competing sources since area was designated less than 10% of the NAAQS |
| Additional Impacts | n/a | Additional impacts analysis * | n/a |
| Alternatives Analysis | n/a | n/a | Alternative sites analysis * |
| Certify Compliance | n/a | n/a | Certify compliance for all sources * |
|  | Source Emissions after the PSEL increase SER to 99 tons per year |  |  |
| Source Classification | Minor | Minor | Minor |
| Preconstruction Monitoring | n/a | n/a | n/a |
| Control Technology | n/a | BACT * | BACT * |
| AQ Analysis-NAAQS/Increments/ AQRV | NAAQS and PSD Increments | NAAQS and PSD Increments | n/a |
|  |  | OR |  |
| NAQB | n/a | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources Source plus competing sources since area was designated less than 10% of the NAAQS | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources Source plus competing sources since area was designated less than 10% of the NAAQS |
| Additional Impacts | n/a | n/a | n/a |
| Alternatives Analysis | n/a | n/a | n/a |
| Certify Compliance | n/a | n/a | n/a |
|  | Source Emissions after the PSEL increase less than SER |  |  |
| Source Classification | Minor | Minor | Minor |
|  | NC rules or ACDP | NC rules or ACDP, no change | NC rules or ACDP, no change |

Reattainment Comparison to Nonattainment and Maintenance

| Program Elements | Nonattainment | Reattainment | Maintenance |
| --- | --- | --- | --- |
|  | Source Emissions after the PSEL increase 100/250 tons per year or more |  |  |
| Source Classification | Federal Major | Federal Major | Federal Major |
| Preconstruction Monitoring | n/a | n/a | Preconstruction Monitoring *** |
| Control Technology | LAER * | LAER * | BACT * |
| AQ Analysis-NAAQS/Increments/ AQRV | AQRV analysis | AQRV analysis | NAAQS and PSD Increments AQRV analysis |
| NAQB | Offsets - 1.1:1 for ozone - 1.2:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources Source plus competing sources since area was designated less than 10% of the NAAQS | Offsets - 1.1:1 for ozone - 1.2:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources Source plus competing sources since area was designated less than 10% of the NAAQS | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources ** and NAQB Source plus competing sources since area was designated less than 10% of the NAAQS Or - Growth allowance Or - Model below maintenance area limits |
| Additional Impacts | n/a | Additional impacts analysis | Additional impacts analysis |
| Alternatives Analysis | Alternative sites analysis * | Alternative sites analysis * | n/a |
| Certify Compliance | Certify compliance for all sources * | Certify compliance for all sources * | n/a |
|  | Source Emissions after the PSEL increase 100 to 99/249 tons per year |  |  |
| Source Classification | Federal Major | Federal Major | Federal Major |
| Preconstruction Monitoring | n/a | n/a | Preconstruction Monitoring *** |
| Control Technology | LAER * | LAER * | BACT * |
| AQ Analysis-NAAQS/Increments/ AQRV | AQRV analysis | AQRV analysis | NAAQS and PSD Increments AQRV analysis |
| NAQB | Offsets - 1.1:1 for ozone - 1.2:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources Source plus competing sources since area was designated less than 10% of the NAAQS | Offsets - 1.1:1 for ozone - 1.2:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources Source plus competing sources since area was designated less than 10% of the NAAQS | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources ** and NAQB Source plus competing sources since area was designated less than 10% of the NAAQS Or - Growth allowance Or - Model below maintenance area limits |
| Additional Impacts | n/a | Additional impacts analysis | Additional impacts analysis |
| Alternatives Analysis | Alternative sites analysis * | Alternative sites analysis * | n/a |
| Certify Compliance | Certify compliance for all sources * | Certify compliance for all sources * | n/a |
|  | Source Emissions after the PSEL increase SER to 99 tons per year |  |  |
| Source Classification | Minor | Minor | Minor |
| Preconstruction Monitoring | n/a | n/a | n/a |
| Control Technology | BACT * | BACT * | BACT * |
| AQ Analysis-NAAQS/Increments/ AQRV | n/a | NAAQS and PSD Increments | NAAQS and PSD Increments |
|  |  | OR | OR |
| NAQB | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources Source plus competing sources since area was designated less than 10% of the NAAQS | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources ** and NAQB Source plus competing sources since area was designated less than 10% of the NAAQS Or - Growth allowance Or - Model below maintenance area limits | Offsets - 1.1:1 for ozone - 1.0:1 for other pollutants, with provision to reduce the ratio if offsets are obtained from priority sources ** and NAQB Source plus competing sources since area was designated less than 10% of the NAAQS Or - Growth allowance Or - Model below maintenance area limits |
| Additional Impacts | n/a | n/a | n/a |
| Alternatives Analysis | n/a | n/a | n/a |
| Certify Compliance | n/a | n/a | n/a |
|  | Source Emissions after the PSEL increase less than SER |  |  |
| Source Classification | Minor | Minor | Minor |
|  | NC rules or ACDP | NC rules or ACDP, no change | NC rules or ACDP, no change |