PFP Mol pavement map key

White lines- Flow line breaks-

Red line- Boundary of project

Orange line- Property fencing

Blue ovals- Existing stormwater detention ponds

Blue arrows- Flow directions

Yellow arrows- Section labeling

CB- Catch basin

Total Project Size = 135,656 SQ Feet

Section A- 95,000 SQ FEET

This section will sheet flow water off of asphalt into existing stormwater detention pond. Pond capacity and infiltration is significant and will not result in a discharge of stormwater into City of Molalla stormwater structures.

Section B- 31,051 SQ FEET

This section has a flow break line to divert water between sections A and section B.  The water from section B will sheet flow into existing catch basin of property. Water from catch basin runs along a vegetated filter strip and then traverses via hard piping to stormwater pond at 300 shaver Ave.

Section C- 9,605 SQ FEET

This section has a flow break line to divert water between section C and section B. Water will sheet flow off of this section into an existing stormwater detention pond. Pond capacity and infiltration is significant and will not result in a discharge into City of Molalla stormwater structures.

Fencing

Property is fenced and on site security is present during unstaffed hours.