[image: image1.jpg]




October 28, 2020

Ms. Alex Liverman
Oregon Department of Environmental Quality

Northwest Region Cleanup Program

700 NE Multnomah Street, Suite 600

Portland Oregon 97232-4100

	Subject
	PGE – Forest Park Property (ECSI #240Annual Progress Report -  October 2020


Dear Alex:  

Pursuant to the Interim Site Management Plan (January 2015), we are submitting two copies of the progress report covering the period through October 2020 for the PGE – Forest Park site.  This report summarizes the accomplishments, plans, data collected, and challenges encountered during this period that commenced following the previous report.  

Actions Taken 

During the current reporting period the following ongoing stormwater management activities have taken place:

1. Vehicle access to the site has remained closed with the installation of eco-block barriers, with the exception of vehicles used to conduct regular maintenance; for example, trucks needed for vegetation maintenance of the PGE powerline corridor accessed via Fire Lane 1; Fire Bureau fire prevention survey crews on ATVs, and City of Portland mowing equipment. 
2. Temporary erosion control has been installed and replaced as needed at the interface of the property and the sidewalk. 
3. Vegetation along the property line and sidewalk was and will continue to be mowed on a regular timeline to help maintain visibility of any potential water draining off the hillside in an effort to identify problem areas.
4. Native vegetation upstream of the catchment basin is in good condition and continues to provide natural filtration. This area will continue to be inspected and if evidence of channelized flow is observed or if there are significant changes to the vegetation in this area, wattles will be replaced to ensure there is adequate filtration.  
5. Unfortunately, the catchment basin itself has repeatedly been used as a bathroom.  Due to the level of illegal camping that is occurring on the site it is difficult to address this ongoing challenge.  
6. The site will continue to be visited during extreme rain events to observe how water is moving throughout the site and identify areas of concern.

7. The Interim Site Management Plan (ISMP) annual reports will continue to be submitted in the fall (by October) each year.     
8. New staff and partners working on the site have been oriented to the ISMP and the guidelines for working on the site prior to conducting any activities on the property.  

Portland Parks & Recreation has been working with consultants to develop a formalized entrance into Forest Park at the subject site.  At this time, PP&R has received approval from Bureau of Development Services on our Land Use Review permit and is moving forward with the proposed development plan.  Construction is anticipated for 2021. The proposed development includes a paved access road and parking lot, repair to Firelane 1 within the project area,, vegetation improvements to the lower wetland and the construction of a lined stormwater facility in the parking lot as well as an overflow and sedimentation catchment inlet in the lower wetland area.   

Because of project delays related to funding and environmental issues, we are requesting an extension of the Consent Order deadline requiring the completion of construction of permanent stormwater features until October 31, 2021. Until then, we will continue to comply with all provisions of the Consent Order and will continue to provide the annual Progress Reports.
Additional Actions Scheduled

· Annual Presentation:  A safety briefing on the ISMP and BMP’s for working on the site was shared at the February 2020 PP&R’s City Nature West safety meeting.
Issues/Resolutions
Routine identified maintenance was completed during this time.  Beyond that body of work, there were no significant issues requiring resolution during this reporting period.
If you have any questions, please call me at 503-823-4492.

Sincerely,

Kendra Petersen-Morgan

Natural Areas Supervisor, City Nature West
Portland Parks & Recreation


cc:
Rachel Felice, City of Portland Parks & Recreation


Cindy Ryals, City of Portland Bureau of Environmental Services

Stacy Hibbard, City of Portland Bureau of Environmental Services

Tina Richards, Office of the City Attorney 

