Blachly Lane Electric Co-op

LUST #20-80-1083 /    DEQ Facility #   / ECSI #    / Q-Time #40704

Address: 90680 HIGHWAY 99 N, Eugene 97402-9624 (Lane County)

Lot #1704050003300

Contact:

- (503) 688-8711

- New RP- Jeff Jones – Operations Manager – 541-580-7209 – jonesj@blec.coop

- accounting@blachlylane.coop

- Past RP – Matt Smith – 541-284-2159

History & Intro:

- My Brief: decommissioned USTs in 1989 left contamination. They installed the new tanks before site characterization. Monitoring water well network was installed. Natural attenuation is occurring but slowly. In 2000 a cNFA was issued with no groundwater use. The decommission of the new tanks in 2008 found that contamination. PCS was removed, but groundwater was still impacted.

  - In 2008 DEQ required a report covering all work in order to review and maybe reconsider the decision. Report = RBC Site Closure of Oct. 2008

- Reported 8/21/2008

- Reported by: Robert DeSpain – Staton Company- 541-726-9422

- Lat 44.1208 N – Long 123.1881 W

- Section 5 Township 17 S Range 4 W

- Airport 1 mile away

- Light industrial and commercial use

- Site mostly flat with elevation of 365 feet.

- Groundwater flow to the north-northwest, confirmed in report from Goodfellow P.6

  - This was using the monitoring wells installed in property

- Decommissioned May 1989 UST

  - By: C & K Petroleum Equipment Company of Eugene

  - (1) 2,500 gallons gasoline

  - (1) 1,000 gallons diesel

  - Located west of fuel shed

- Installed September 1989 UST

  - (1) 6,700 gallons diesel

  - (1) 2,300 gallons gasoline

  - New pit also west fuel shed

  - Decommissioned by Staton Companies in August 2008

- Database comments:

  - They think this is the remains from an old release, from an earlier NFA that Robert Goodfellow investigated.  However, they had some sawdust underneath the dispenser, so I suspect there might have been a spill/release there some time in the past.  However, that was not part of the contamination that they observed in the field.  They did not dig up the dispenser, nor take samples the day I was out there.  They did pump water out of the excavation and planned to go out there the next day and sample.  I did not observe the actual sampling, nor see how extensive any spill might have been near the dispenser (which was less than 10 feet from the tank, but at the opposite end of the tank from where they observed most of the contamination)... tmd 8-25-08.

  - The tanks we removed Thursday were fairly new and were not the source of the release.  Previous tanks that were removed in 1989 were the cause of the release.  Robert DeSpain 8/25/08.

  - See also - 20-89-4096 Blachly Lane Electric

Findings:

- Tank Removal and Cleanup Report - October 2008

  - Staton Companies – Demo – Envr. Work

  - Prepared by Rober DeSpain

  - Contracted to decommission 2 tanks and one dispenser on August 21, 2008

  - (1) 6,700 gallons diesel and (1) 2,300 gallons gasoline

  - Excellent conditions / not the cause

  - PCS encountered around 6’6” bgs north side of gasoline tank

  - September 29, 2008 – 100 cubic yard of PCS disposed at Coffin Butte Landfill

  - Groundwater encountered around 11’ bgs

  - 330 gallons of oily water and free product disposed of ORCCO, Eugene

  - Contamination presumed to be from 1980’s USTs at site

- RBCA Site Closure Report – October 24, 2008

  - Goodfellow Environmental and Geological Services

  - After decommissioning of USTs in 1989 some contamination was left

  - DEQ sent letter in August 1991 for further action

  - In October 1991 soil and GW investigation around tank pit area

  - Contamination found – mostly gasoline – around pit and to the north

  - Extent confirmed around pit and to the north by numerous borings advanced

  - 2 zones: 1) shallow – gasoline – 12-15’ to the north (likely surface spill)

  - 2) deeper – 9-10’ bgs in soil-water interface

  - GW contaminated with gasoline ~ 36-50’ to the north

  - 1992 – Corrective Action Plan submitted and approved

    - Excavation of PCS

    - Treatment GW by extracting air stripping - never installed

    - ~350 cubic yards of PCS removed – disposed Short Mt. Landfill, Goshen

    - 3 monitoring wells and (1) 6” recovery well

    - Plume is situated below paved parking lot

  - GW Monitoring Network

    - Installed August 1992

    - First report October 1992 – result lower BTEX – in-situ degradation

    - Quarterly monitoring from 1993 – 1994

    - Bi-annual and annual 1995 – 2000

    - Last – April 2000

    - Contaminants lowered through time

  - Closure 2000

    - Due to new RBDM rules site could be closed

    - Report from RP submitted May 2000

    - Ensure cap of water well at site

    - November 2000 issued NFA

      - No future water wells

  - 2008 Decommissioning

    - tanks, pipes and dispenser in good condition

    - PCS soils were found in pit

    - PCS were removed, ~ 100 cubic yards – August 2008

    - Groundwater entered pit, was purged and wait 24 hours

    - Confirmation samples taken including GW

    - Results prove ok and contamination is assumed to be from previous tanks

Conclusion:

- Site remains the same

- Receptor = Occupational

- Only main exceedance “Ingestion & Inhalation from Tap Water”

- Site is connected to city water supply

- No wells in area

- No new USTs

Questions: