General Requirements (OAR 340-049-0015) - Each owner of a regulated wastewater system must have its system supervised by one or more operators who hold a valid certificate for the type of system, wastewater treatment or collection, and at a grade equal to or greater than the wastewater system classification as defined in OAR-340-049-0020 and 0025.  DEQ will advise system owners of the classification of their systems as a permit action.  As the classification establishes the operator certificate type and grade required for compliance, it needs to be set prior to “start-up” of a new or upgraded and/or expanded facility.

Wastewater treatment system classifications will be derived from the total points assigned based on criteria shown in OAR 340-049-0025 (see Classification Worksheet).  Collection system classifications are based on design population or population equivalent to be served by a wastewater treatment system (see Worksheet).

Upon written notice to the wastewater system owner, DEQ may classify a wastewater treatment system higher than the classification based on accumulated points if the complexity of a treatment system is not reflected in the criteria(see Worksheet examples).  If deemed appropriate, DEQ may classify a wastewater collection system higher than the classification based on population when a Class I by population will have significant pumping of sewage including STEP or other pumping that may warrant a Class II designation.  In either case, designation must be consistent with the intent of the classification system (see OAR 340-049-0020(4) & (5)).

Classification of Wastewater Systems (OAR 340-049-0020)   All wastewater systems regulated under OAR 340-049 will be classified by DEQ as wastewater treatment systems and/or wastewater collection systems, as appropriate, in accordance with the following classification system:

| Wastewater Treatment Systems | Wastewater Collection Systems |
| --- | --- |
| Class I - 30 total points or less | Class I - 1,500 or less design population |
| Class II - 31-55 total points | Class II - 1,501 to 15,000 design population |
| Class III - 56-75 total points | Class III - 15,001 to 50,000 design population |
| Class IV - 76 or more points | Class IV - 50,001 or more design population |

Definitions used in these regulations unless otherwise required by context (see OAR 340-049-0010):

"Average Dry Weather Flow" (ADWF) means the design average dry weather flow capacity of the wastewater treatment system in gallons per day or Million Gallons per Day (MGD), as approved by the Department.

"Industrial Waste" means liquid wastes from an industrial or commercial process discharged into a wastewater system for conveyance and treatment.

"NPDES Permit" means a waste discharge permit issued in accordance with requirements and procedures of the National Pollutant Discharge Elimination System authorized by Section 402 of the Federal Clean Water Act and OAR 340, Division 45.

"Population" means the design population of the wastewater system represented as the number of people or the population equivalent the system is designed to serve. Equivalent population ordinarily is determined based on 70 gallons per person per day average dry weather flow (ADWF) or 0.17 lbs. BOD5 per person per day, whichever is greater.

"Wastewater" or "sewage" means the water-carried human or animal waste from residences, buildings, industrial establishments or other places, together with such groundwater infiltration and surface water as may be present. The admixture of domestic and industrial waste or other by-products, such as sludge, is also considered wastewater or sewage.

"Wastewater Treatment System" or "Sewage Treatment System" means any structure, equipment or process for treating and disposing of, or recycling or reusing wastewater and sludge (including industrial waste) that is discharged to the wastewater system.

"Wastewater Collection System" or "Sewage Collection System" means the trunks, arterials, pumps, pump/lift stations, piping and other appurtenances necessary to collect and carry away wastewater or other liquid waste treatable in a community or private wastewater treatment facility.

"Wastewater System" means "Sewage Treatment Works" defined in ORS 448.405 as any structure, equipment or process required to collect, carry away and treat domestic waste and dispose of sewage as defined in ORS 454.010. Typically, components of a wastewater system include a wastewater collection system and a wastewater treatment system.

"WPCF Permit" means a Water Pollution Control Facilities permit to construct and operate a collection, treatment and/or disposal system with no discharge to navigable waters.

WW System Common Name: City of Newberg STP

Facility ID: 102894		Location: 2301 Wynooski Street, Newberg

Total Points (from page 3): 107.5		WWT Class (check):	 I	 II	 III	 IV

Design Population: 45000		WWC Class (check):	 I	 II	 III	 IV

Design ADWF load (Influent MGD) 4.0		Design BOD load (Influent lbs./day) 7200

Classified by: Mark E. Hamlin		Date: April 26, 2004

Date this classification filed with the Operator Certification office:

System start-up date for this classification (new, upgrade or expansion): 1987

Is this a change from a prior classification? (check):	 Yes	No

Criteria for Classifying Wastewater Treatment Systems (OAR 340-049-0025)

(1)	Design Population or Population Equivalent Points (10 Points Maximum)

Less than 750						0.5 points

751 to 2000									1 point

2001 to 5000									1.5 points

5001 to 10,000								2 points

Greater than 10,000	3 points plus 1 per 10,000

Point subtotal	7.5

(2)	Average Dry Weather Flow (Design Capacity) Points (10 points Maximum)

Less than 0.075 MGD							0.5 point

Greater than 0.075 to 0.1 MGD						1 point

Greater than 0.1 to 0.5 MGD						1.5 points

Greater than 0.5 to 1.0 MGD						2 points

Greater than 1.0 MGD 							3 points plus 1 per 1 MGD

Point subtotal	7.0

- Unit Process Points  (Check all that apply)

Preliminary Treatment and Plant Hydraulics:

Comminution (includes shredders, grinders, etc.)				1 point

Grit Removal, gravity							1 point

Grit Removal, mechanical							2 points

Screen(s), in-situ or mechanical						1 point

Pump/Lift Station(s) (pumping of main flow)				2 points

Flow Equalization (any type)						1 point

Point subtotal		7.0

Primary Treatment:

Community Septic Tank(s)							2 points

Clarifier(s)									5 points

Flotation Clarifier(s)								7 points

Chemical Addition System							2 points

Imhoff Tank (or similar)							3 points

Point subtotal		0

Total Points Page 1	21.5

Page 1 of 3

Unit Process Points – Continued (Check all that apply)

Secondary, Advanced, and Tertiary Treatment:

Low Rate Trickling Filter(s) (no recirculation)				7 points

High Rate Trickling Filter(s) (recirculation)			10 points

Trickling Filter - Solids Contact System					12 points

Activated Sludge (any type)							15 points

Pure Oxygen Activated Sludge						20 points

Activated Bio Filter Tower less than 0.1 MGD				6 points

Activated Bio Filter Tower greater than 0.1 MGD 				12 points

Rotating Biological Contactors 1 to 4 shafts				7 points

Rotating Biological Contactors, 5 or more shafts				12 points

Stabilization Lagoons, 1 to 3 cells without aeration				5 points

Stabilization Lagoons, 1 or more cells with primary aeration		7 points

Stabilization Lagoons, 2 or more cells with full aeration			9 points

Recirculating Gravel Filter							7 points

Chemical Precipitation Unit(s)						3 points

Gravity Filtration Unit(s)							2 points

Pressure Filtration Unit(s)							4 points

Nitrogen Removal, Biological or Chemical/Biological System		4 points

Nitrogen Removal, Designed Extended Aeration Only			2 points

Phosphorus Removal Unit(s)						4 points

Effluent Microscreen(s)							2 points

Chemical Flocculation Unit(s)						3 points

Chemical Addition System(s) (6 points maximum)	@  2 points

Point subtotal		15

Solids Handling:

Anaerobic Primary Sludge Digester(s) w/o Mixing and Heating		5 points

Anaerobic Primary Sludge Digester(s) with Mixing and Heating		7 points

Anaerobic Primary and Secondary Sludge Digesters			10 points

Sludge Digester Gas reuse							3 points

Aerobic Sludge Digester(s)							8 points

Sludge Storage Lagoon(s)	(or tanks, basins etc.)				2 points

Sludge Lagoon(s) with aeration						3 points

Sludge Drying Bed(s)							1 point

Sludge Air or Gravity Thickening						3 points

Sludge Composting, In Vessel						12 points

Sludge Belt(s) or Vacuum Press/Dewatering				5 points

Sludge Centrifuge(s)								5 points

Sludge Incineration								12 points

Sludge Chemical Addition Unit(s) (alum, polymer, etc.)			2 points

Non-Beneficial Sludge Disposal						1 point

Beneficial Sludge Utilization							3 points

Point subtotal		25

Disinfection:

Liquid Chlorine Disinfection							2 points

Gas Chlorine Disinfection							5 points

Dechlorination System							4 points

Other disinfection systems incl. ultraviolet and ozonation			5 points

Point subtotal		9.0

Total Points Page 2	49

Page 2 of 3

(4)	Effluent Permit Requirement Points (Check as applicable):

Minimum of secondary effluent limitations for BOD and/or TSS		2 points

Minimum of 20 mg/L BOD and/or Total Suspended Solids			3 points

Minimum of 10 mg/L BOD and/or Total Suspended Solids			4 points

Minimum of 5 mg/L BOD and/or Total Suspended Solids			5 points

Effluent limitations for effluent oxygen					1 point

Point subtotal		4.0

(5)	Variation in Raw Waste Points. (6 points maximum)  Points in this category will be awarded only when conditions are extreme to the extent that operation and handling procedure changes are needed to adequately treat waste due to variation of raw waste

Recurring deviations or excessive variations 100% to 200% 		2 points

Recurring deviations or excessive variations of more than 200% or

conveyance and treatment of industrial wastes by Pretreatment program	4 points

Septage or other hauled waste (control and/or preliminary treatment)	2 points

Point subtotal		6.0

- Sampling and Laboratory Testing Points (check as applicable - maximum 11 points)

Sample for BOD, Total Suspended Solids performed by outside lab	2 points

BOD or Total Suspended Solids analysis performed at treatment plant	4 points

Bacteriological analysis performed by outside lab				1 point

Bacteriological analysis performed at WWT plant lab			2 points

Nutrient, Heavy Metals or Organics analysis performed by outside lab	3 points

Nutrient, Heavy Metals or Organics analysis performed at WWT plant	5 points

Point subtotal		11

- Points For Other Complexities Not Reflected Above: (see OAR 340-049 0020(4) & (5))

Odor Control (2 points maximum)						1 to 2 points

Standby Power Units 	@  1 point

Solids Composting or Land Application of Biosolids			10 points

Alkaline Stabilization (3 points maximum)					2 to 3 points

Other Effluent Limits [ammonia, Cl2, temp., etc. (list or attach list)]	@  1 point

Pond(s) (advanced treatment polishing or irrigation holding)		2 points

Effluent Land Disposal - Evaporation (surface or subsurface)		2 to 4 points

Effluent direct Reuse or Recycle 						6 points

SCADA or similar for data (limited to extensive total process operation)	2 to 6 points

Chemical/Physical advanced waste treatment following secondary	10 points

Chemical/Physical advanced waste treatment w/o secondary		15 points

Biological or Chemical/Biological advanced waste treatment		12 points

Reverse Osmosis, Electro-dialysis or Membrane Filtration techniques	15 points

Other complexities (list or attach list):

Point subtotal		16

Total Points Page 3	37

Total Accumulated Points (3 pages)	107.5

A COPY OF THIS COMPLETED WORKSHEET IS TO BE FILED WITH THE OPERATOR CERTIFICATION PROGRAM, WATER QUALITY DIVISION, PRIOR TO SYSTEM START-UP

Page 3 of 3

See “Population” definition.  Use the design average daily per person load for Influent Flow or Influent BOD5, whichever is greater.  This value is also used to determine the Collection System Classification.