# October 4, 2025

Banks School District

13050 NW Main Street

Banks, Oregon, 97103

RE:	No Further Action Determination
for Banks High School

Banks, Oregon

Leading Underground Storage Tank (LUST) Program No. 34-14-0891

To Whom it May Concern:

The Oregon Department of Environmental Quality (DEQ) has completed a review of the available information for the Banks High School site/property, including the closure report entitled Unregulated Gasoline Tank Decommissioning and Risk-Based Closure Report dated August 25, 2014, which was submitted to DEQ by EVREN Northwest Inc. (ENW) on your behalf. The Banks High School address is 13050 Northwest Main Street, Banks, Oregon, 97103, Tax Lot 6900 of Washington County Map No. 2N331BC.

DEQ has determined that remedial action to address environmental contamination at Banks High School is complete, and no further action is required. This determination is a result of our evaluation and judgment based on the DEQ regulations and the facts as we now understand them including the following:

- The 15.32 acre Site is developed with the Banks High School, including several school buildings and athletic facilities. The Site is bordered to the north by residential and commercial properties; to the west by NW Main Street with residential properties and agricultural land beyond; to the south by athletic fields with residential and commercial properties beyond; , and to the east by residential properties with agricultural land beyond.. The Site is likely to continue to be used as a high school in the foreseeable future.

- A 1,050-gallon underground storage tank was discovered on July 3, 2014, during demolition of an existing grounds keeping building, which was located 80 feet west of the track and field facility and 18 feet south of the track and field grandstands. The tank was located approximately 2.5 feet below ground surface.

- Laboratory analytical results determined that soils adjacent to the tank were impacted at up to 3,400-milligrams per kilogram gasoline-range organics (GRO), exceeding DEQ’s Soil Matrix Level 2 cleanup level. Approximately 596 tons of impacted soil were excavated and disposed of at Hillsboro Landfill. The excavation was completed to a depth of 10.5 feet.

- Groundwater was observed seeping into the tank excavation at an approximate depth of 10.5 feet. A groundwater (pit water) sample was collected after evacuating the pit of water and allowing it to recover. No GRO or related volatile organic compound (VOC) constituents were detected. Confirmation soil samples were collected from the excavation floor and sidewalls. All confirmation samples were “nondetect” for GRO VOCs, and total lead.

- Future groundwater contamination associated with the UST petroleum release is not expected because all impacted soils have been removed and the concentrations of GRO and VOCs in the source-area reconnaissance groundwater sample did not exceed laboratory reporting limits.

- There is no risk of exposure to Site contamination by human or ecological receptors because all contaminated soils have been removed.

Based on the available information, soil and groundwater conditions at Banks High School are currently protective of public health and the environment in accordance with Oregon Administrative Rules 340-122-0205 through 340-122-0360. The site requires no further action unless new or previously undisclosed information becomes available, or there are changes in site development or land and water uses, or more contamination is discovered. DEQ will update the Your DEQ Online (YDO) database to reflect this decision.

This letter only applies to the release(s) discussed above. If any contaminated media is encountered in the future, it must be handled and disposed of in accordance with local, state and federal regulations.

Documents supporting this No Further Action decision, including the Unregulated Gasoline Tank Decommissioning and Risk-Based Closure Report, can be viewed on the Your DEQ Online (YDO) database (YDO Link). DEQ recommends keeping a copy of all of the documentation associated with this remedial action with the permanent facility records. If you have any questions, please contact Deb Goldberg at (971) 985-9391, or via email at deb.goldberg@deq.oregon.gov.

Sincerely,

Peter Donahower, Manager

Northwest Region Petroleum Cleanup Section

Attachment(s): 	Figure 4 – Sample Location Diagram

Tabel 2 – Summary of Analytical Data, Reconnaissance Ground Water Recharge

cc:

EVREN Northwest Inc.

LUST #34-14-0891 File