May 8, 2006
EAST MULTNOMAH COUNTY/CASCADE CORP

DEQ Approval of Changes to Treatment System

	CHANGE REQUEST
	APPROVAL DATE
	FORMAT
	COMMENTS

	Cascade: Modify existing TGA trench from extraction to passive bio and extend trench to east.
	10/13/05
	DEQ letter
	Trench modification work plan reviewed and approved. Work Plan dated 8/31/05.

	EMC: Decommission well MW-26ds on Fudge property.
	11/7/05
	Email*
	Compliant for 3 years, request by property owner to facilitate property development. Plan dated 10/13/05.

	EMC: Conduct diffusion bag sampling alongside regular well monitoring to assess shift to use of diffusion bag samplers.
	11/15/05
	Email
	Reviewed diffusion bag sampler work plan and approved via email. Plan dated 11/8/05.

	EMC: Conduct second round of diffusion bag sampling during May monitoring event to evaluate result differences for some wells.
	3/23/06
	Email
	Re-sample all wells but differences noted with only three wells. Email request from Sara Prowell 3/23/06.

	EMC: Reduce monitoring from quarterly to semi-annual for pilot shutdown of wells EW-4, EW-5, EW-6, EW-17, and RPW-2.
	3/13/06
	Email
	Two years of sampling w/ all but EW-5 declining and/or below cleanup levels, EW-5 exceeds only twice. Email request from Sara Prowell 3/10/06.

	Cascade: Implement bioremediation work plan for Source Areas 1 and 2.
	5/1/06
	Email
	Reviewed Bioremediation Work Plan and approved via email. Follow up needed with UIC program. Plan dated 3/24/06. UIC registration form for 9 injection wells to DEQ dated 5/11/06.

	EMC and Cascade: Discontinue sampling for phosphate and nitrogen constituents and lead in treatment system discharge effluent under EMC and Cascade Orders.
	5/2/06
	Email
	8 years of sampling have sufficiently characterized these constituents. If a need arises in the future to track these constituents for compliance then DEQ and RPs will revisit sampling program. Email request from Sara Prowell 3/10/06.

	EMC: Work Plan for Groundwater Extraction Well Pilot Shutdown of EW-7, EW-9, EW-19, and EW-20, TSA Remedial Action
	6/2/06
	Email
	Two-year shutdown & monitoring w/ result of decommissioning wells. Contingency of restart pumping. New well EW-23 work plan to be submitted.

	BOP: T-18 Spill Initial Response Investigation Work Plan
	6/5/06
	Email
	Soil sampling around tank, groundwater sampling from nearest down gradient wells. 

	Cascade: Shut Down of Area 1 & 2 Extraction for Air Stripper Cleanup
	6/5/06
	Verbal
	During site visit to observe start up of bio injection for Area 1, Sarah Prowell requested continued shut down of extraction wells after injection to perform maintenance on air stripper unit (a day or so of extra shut down).

	BOP: T-18 Spill Supplemental Investigation Work Plan
	6/15/06
	Email
	Coring through tank at four side locations and one bottom location. Sample backfill and auger to native soil.

	EMC: Decommission wells EW-6, EW-20, DEQ-3i, DEQ-3usg, and DEQ-3d due to City of Gresham WWTF expansion
	8/29/06
	Letter
	Decommission the wells in place by pressure grouting, monitoring picked up by other nearby wells

	BOP: Oil spill in soil in building, cleanup report forthcoming
	8/24/06
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	Notes: (*) cannot locate copy of letter or email


EMC/Boeing/Cascade System Changes

June 20, 2006 Update

Page 1 of 2

