East Multnomah County Groundwater Cleanup Sites

Webpage on DEQ Website

July 26, 2010

The following information provides the basis for creation of an EMC cleanup project webpage that will consolidate an update on progress for cleanup of the EMC site and the related Cascade Corporation and The Boeing Company sites. The purpose of the webpage is to begin alerting stakeholders that these sites are nearing their cleanup goals, although at somewhat different timing. The reason a webpage was considered important is that the history of this site has been somewhat contentious with multiple parties responsible for the work, its location within (and threatening) the City of Portland South Shore Well Field, lawsuits and threatened lawsuits between/among the parties and the City, many area residents and business affected by the presence of contamination in their wells and beneath their property, and limitations the contamination has imposed on use of the well field. The goal is to inform stakeholders early on, before we reach a closeout point, to be able to know about and address any concerns with these projects.

Here is the information you need to build the EMC webpage based upon headings in other existing project webpages.

- The title of the webpage: East Multnomah County Groundwater Cleanup Sites

- ECSI numbers: 1479 (East Multnomah County), 635 (Cascade Corporation), 13 (The Boeing Company)

- Project Manager: Bruce Gilles (Interim)

- Phone: 503-667-8414 X55009

- Email: gilles.bruce@deq.state.or.us

- Site Summary: Groundwater within the Troutdale Sandstone Aquifer in East Multnomah County has been contaminated with industrial solvents that leaked or spilled into the aquifer from the Cascade Corporation and Boeing properties. The contaminated portion of the Troutdale Sandstone Aquifer lies within the eastern part of the City of Portland South Shore Well Field near Gresham Oregon. Originally, the solvents entered shallow groundwater of the Troutdale Gravel Aquifer beneath both properties, then migrated downward and offsite into the Troutdale Sandstone Aquifer. Since DEQ selected remedies for these aquifers in 1996 and 1997, Boeing and Cascade have been working to cleanup groundwater in the Troutdale Gravel Aquifer on their properties and jointly cleanup groundwater in the Troutdale Sandstone Aquifer. More recently, Boeing and Cascade have begun to closely evaluate cleanup progress and to implement additional cleanup measures to accelerate achieving their cleanup goals. {“More”}

- Documents:

EMC: “Semiannual Performance Report, October 1, 2009 through March 31, 2010, Troutdale Sandstone Aquifer Remedy”, Prowell Environmental and Landau Associates, May 28, 2010

Cascade Corporation: “2009 Year-End Semiannual Performance Report, Troutdale Gravel Aquifer: Reporting Period July through December 2009”, Prowell Environmental, March 2, 2010

“Final 2009 Mid-Year Semiannual Performance Report and Pre-Closure Plan, Troutdale Gravel Aquifer: Reporting Period January through June 2009”, Prowell Environmental, October 6, 2009

“Cascade Corporation Troutdale Aquifer Cleanup Progress Update August 2010”, DEQ Fact Sheet

“Updated Cascade Corporation TGA Remedy Pre-Closure Plan”, Letter from Prowell Environmental for Cascade Corporation dated July 26, 2010

The Boeing Company: “2009 Progress and Performance Evaluation Troutdale Gravel Aquifer, Boeing Portland Facility, Gresham, Oregon”, Landau Associates, March 18, 2010

“Draft Remedy Completion Strategy Plan, Troutdale Gravel Aquifer, Portland, Oregon”, Landau Associates, March 31, 2010.

“Report, Remedial Cleanup Action Building 85-001 Expansion, Boeing Portland, Gresham, Oregon”, Landau Associates, July 6, 2010

“Environmental Subsurface Investigation Building 85-001 Former Degreaser Area, Boeing Portland Facility, Portland, Oregon”, Landau Associates, July 22, 2010