East Multnomah County TSA and SGA Remedy Timeline

| Action |  | Date |  |
| --- | --- | --- | --- |
| Corrective Measures Element |  |  |
| Entered into Order on Consent with DEQ for TSA Removal Action to control northerly plume migration (DEQ No. ECSR-NWR-93-07) | 1993 |  |
| Installed RPW-1 and RPW-2 and began extracting RPW-2 and treating effluent | 1994 |  |
| Completed TSA RI/FS |  |  |
| Entered into Addendum to Order on Consent to complete TSA RI/FS | 1994 |  |
| Entered into Second Addendum to Order on Consent for NPDES discharge permit | 1996 |  |
| Performed and completed TSA RI/FS, including well installations, aquifer testing, model construction, evaluations, design planning, and reporting | 1994 - 1996 |  |
| DEQ issued TSA Remedy Record of Decision | 1996 |  |
| Enter into Order on Consent with DEQ for design and implementation of remedial measures (DEQ No. WMCSR-NWR-96-08) | 1997 |  |
| Implemented TSA Remedy |  |  |
| Installed SGA monitoring wells to characterize water quality; decommissioned selected private SGA wells for migration pathway protection; provided replacement water supply to residences | 1996-1997 |  |
| Performed phased remedy implementation including: construction of monitoring and extraction wells (EW- 1, EW-2, EW-3), convert monitoring wells MW-16 and MW-24 to extraction wells EW-4 and EW-5, install conveyance and treatment systems, and performance/protection evaluations | 1996-1998 |  |
| Began phased startup of remedy extraction/treatment systems, installation of extraction wells EW-6, EW- 7, EW-8, EW-9, EW-10, EW-11, EW-12, EW-13, EW-14, EW-15, EW-16, EW-17, EW-18, EW-19, and EW-20 | 1996-1998 |  |
| Entered into Order on Consent with DEQ for remedy implementation and effluent discharge | 1997 |  |

| Action | Date |
| --- | --- |
| Began full scale remedy implementation of GWTS (West), Central, East, North, and Far North Systems | 1998 |
| Began operating EW-21 Treatment System for TSA Sandstone plume capture near MW-26(ds) | 2000 |
| Amended TSA RD/RA work plan and documented transfer of TSA remedy responsibilities from Boeing to Cascade | 2001 |
| Began operating EW-22 for TSA Sandstone plume capture near BOP-22(ds) | 2003 |
| Began operating EW-23 from Lower TSA | 2007 |
| Began Remedy Modifications, Due to Restoration Progress |  |
| Decommission East Treatment System | 2005 |
| Decommission Far North System | 2007 |
| Decommission North Treatment System | 2008 |
| Adopt diffusion bag samplers methods for groundwater sampling, pilot shutdown of EW-7, EW-9, EW-19, and EW-20; and installation of TSA Sandstone well EW-23 | 2006 |
| Decommission BOP-22(ds) and install replacement well BOP-22R(ds), install BOP-70(ds) and BOP- 71(ds) | 2008-2009 |
| Shutdown EW-22 | 2008 |
| Pilot Shutdown EW-3 and EW-13 | 2009 |
| Cessation of TSA treatment at Groundwater Treatment System (GWTS - West) | 2009 |
| Install replacement well BOP-60R(ds) and decommission BOP-60(ds) | 2010 |
| Decommission EW-22 | 2010 |
| Cascade Corporation receives NFA for TGA | 2015 |
| Begin SVE operation in TSA Mound Area | 2015 |